|  | Huawei IAD Feature Description |  |  |  |
| --- | --- | --- | --- | --- |
|  | Issue | 2.1 |  |  |
|  | Date | 2017-05-02 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2017. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | http://www.huawei.com |
| Email: | support@huawei.com |

About This Document

Change History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 2012-05-28 | V1.0 | Completed the initial draft. | Si Yuanhai/00163019 |
| 2013-09-15 | V2.0 | Add IAD196 | Yang Yijin/51270 |
| 2015-05-30 | V2.1 | - Delete the IAD102 and IAD208 products from the document - Delete the G.723 from the document - Delete the 'Remote Packet Capturing' feature from the document | Yang Yijin/00266318 |

Contents

About This Document	ii

1 Port Description	1

1.1 Uplink Ethernet Ports	1

1.2 Downlink Ethernet Ports	1

1.3 FXS	1

1.4 FXO	1

1.5 Survival Ports	1

1.6 Serial Port	2

2 Signaling and Protocols	1

3 Voice Quality	2

3.1 Voice Codecs	2

3.2 VAD	2

3.3 CNG	2

3.4 Echo Cancellation	3

3.5 Jitter and Disorder Prevention	3

3.6 Gain Adjustment	3

3.7 Detection and Generation of DTMF Code	3

3.8 DTMF Transmission	3

3.9 QoS Guarantee	4

4 Services	5

4.1 Automatic configuration using DHCP	5

4.2 Registration	5

4.3 RTP/RTCP Packaging	5

4.4 Fax	5

4.5 Dual-Homing	6

4.6 Cascading Function	6

4.7 Hotline Services	6

4.8 Local Switch	6

4.9 Local Switch When Network Connection Is Normal	6

4.10 Remote Packet Capturing	7

4.11 Polarity Reversal Charging	7

4.12 Video Call Fallback	7

4.13 MWI	7

4.14 Power-off Survival	7

4.15 Network Interruption Survival	7

4.16 Session Authentication	8

4.17 Hookflash Function	8

4.18 Three-Party Services	8

4.19 Emergency Call	8

4.20 Digitmap Function	8

4.21 Multi-Country Adaptation	9

4.22 Phone Port Properties	9

4.23 VLAN Function	9

4.24 VQM Function	9

4.25 Signaling and Media Encryption	9

4.26 Service List (with IMS)	10

4.27 Service List (with NGN)	11

4.28 Service List (with U1900)	12

# Port Description

## Uplink Ethernet Ports

UC integrated access device (IAD) registers with a Session Initiation Protocol (SIP) server or media gateway controller (MGC) through a 10/100 Mbit/s uplink Ethernet port (RJ45-type).

## Downlink Ethernet Ports

The downlink network port of IAD104H can be connected to a PC. IAD132 downlink network port is used as IAD132 cascade. IAD1224/IAD196 provides two downlink FE ports. One port (LAN port) is used for debugging (the port supports only 100 Mbit/s and is unavailable when the remote packet capturing function is enabled), and the other port is used for cascading.

## FXS

IAD connects to plain old telephone service (POTS) phones or fax machines through Foreign Exchange Station (FXS) ports.

## FXO

IAD connects to a public switched telephone network (PSTN) through a Foreign Exchange Office (FXO) port.

## Survival Ports

FXO ports that support the power-off survival function connect to FXO ports physically. When IAD encounters a power failure, POTS phones connected to these FXO ports can still make or answer PSTN calls.

## Serial Port

IAD provides an RS232 serial port. Users configure and maintain the device through the serial port.

# Signaling and Protocols

IAD complies with the following signaling and protocols:

- Tone detection and generation

- Dual tone multiple frequency (DTMF) and call tones (such as dialing tone, busy tone, ring back tone, and off-hook tone)

- International Telecommunication Union Telecommunication Standardization Sector (ITU-T) V.17, V.21, V.27ter, and V.29 fax standards

- Voice/multimedia digital signal coding and decoding

- ITU-T G.711 µ-law (64 kbit/s)

- ITU-T G.711 A-law (64 kbit/s)

- Internet Low Bitrate Codec (iLBC) (supported only by IAD1224/IAD196)

- ITU-T G.729

- Call control

- SIP (RFC3261–3265)

- Media Gateway Control Protocol (MGCP) (RFC3435)

- Voice packet encapsulation and decapsulation

Real-Time Transport Protocol (RTP)/Real Time Transport Control Protocol (RTCP) (RFC1889)

- Internet

- Transmission Control Protocol (TCP)/Internet Protocol (IP)

- User Datagram Protocol (UDP)/IP

- Address Resolution Protocol (ARP)/(Reverse Address Resolution Protocol) RARP

- Internet Control Message Protocol (ICMP)

- Telnet/Secure Shell (SSH)

- Domain Name Server (DNS) client

- File Transfer Protocol (FTP)/Trivial File Transfer Protocol (TFTP)/File Transfer Protocol Secure (FTPS) client

- IP address allocation

- Static

- Dynamic Host Configuration Protocol (DHCP) client

- Point-to-Point Protocol over Ethernet (PPPoE)

# Voice Quality

## Voice Codecs

IAD supports the following voice codecs:

- G.711 64 kbit/s (lossless compression)

- G.729 8 kbit/s (lossy compression)

- iLBC

iLBC has better performance than G.729, and provides a special mechanism for processing lost packets. The voice quality can be ensured even in a network environment with a higher packet loss rate. Only IAD1224/IAD196 supports iLBC.

You can select the codec based on your requirements, for example, high encoding rate, good voice quality, or high bandwidth.

## VAD

The voice activity detection (VAD) technology recognizes non-speech sections in a voice signal stream.

Based on statistics, during a voice call, silent periods account for 40% of the time. IAD automatically detects silent periods and stops transmitting data during silent periods, reducing the voice data volume.

## CNG

Daily voice communication is conducted in an environment with more or less noise. When the VAD technology is applied, silent periods that are absolutely quiet will make calling and called parties feel uncomfortable. IAD uses the comfort noise generation (CNG) technology to generate appropriate background noise during silent periods to increase the comfort of users.

## Echo Cancellation

Echo is classified into acoustic echo and hybrid echo.

Acoustic echo arises when sound from a loudspeaker is picked up by the microphone in the same room.

Hybrid echo is generated by the PSTN through the reflection of electrical energy by a device called a hybrid. Most telephone local loops are two-wire circuits while transmission facilities are four-wire circuits. Each hybrid produces echoes in both directions.

The echo cancellation technology uses a predictive filter to recognize echo in the transmitted or received signal based the reference signal, and removes the echo.

The echo cancellation technology used by IAD complies with the G.168 standard. The echo cancellation duration for IAD1224/IAD196 is longer than 64 ms. The echo cancellation duration for other IADs is longer than 32 ms.

## Jitter and Disorder Prevention

Jitter occurs when the rate of data packets changes due to latency. Jitter causes disorder in a packet sequence. To reduce voice distortion caused by jitter and disorder, IAD uses a jitter buffer store packets for a specified period of time to ensure that the slowest packet can arrive in time. IAD adjusts the message length sent to the voice processing module to ensure that voice data is sent the voice processing module with uniform speed.

## Gain Adjustment

IAD automatically performs gain adjustment on voice volume to ensure the volume is appropriate. The gain adjustment includes input gain (IP > pulse code modulation (PCM)) and output gain (PCM > IP) adjustment.

## Detection and Generation of DTMF Code

IAD provides the DTMF detection function and generates DTMF codes that comply with international standards.

## DTMF Transmission

IAD supports RFC2833 (RTP load format specifically for DTMF digital signals, phone tones, and call signals). IAD also supports RFC2833 encryption to prevent DTMF code to be stolen on unsafe IP networks.

## QoS Guarantee

IAD supports IP Precedence/differentiated services code point (DSCP) labels. Media, signaling, and network management packets can be assigned with high-priority labels.

IAD supports 802.1P/Q. Signaling, media, and network management virtual local area networks (VLANs) can be created based on the IP address of the same physical port. VLAN priorities can be configured.

IAD marks voice traffic by setting DSCP values in the IP layer or adding VLAN tags in the data link layer. Forwarding devices perform different operations on packets based on the tags to ensure that voice packets with higher priorities are forwarded first.

IAD supports the Voice Quality Monitor (VQM) function, which monitors the voice quality in real time and sends the quality information to a data center.

# Services

This section describes voice services under U1900, next generation network (NGN), or IP multimedia subsystem (IMS).

## Automatic configuration using DHCP

After being connected to the network, IAD obtains its own IP address through DHCP and obtains the IP address of the TFTP server. Then IAD obtains the corresponding configuration file from the TFTP server. Users can use the updated IAD configurations without configuring IAD.

Configuration files must be put on the TFTP server in advance.

## Registration

Users under IAD can be registered with a softswitch (such as SoftX3000 or U1900) using SIP or MGCP to implement voice services. IAD also can be registered with the IMS to implement voice services, which include basic call and fax services.

## RTP/RTCP Packaging

IAD supports RTP and RTCP packets. The packaging duration can be 10 ms, 20 ms, or 30 ms.

## Fax

IAD supports T.30 faxes in the circuit switched domain and T.38 faxes in the packet switched domain, and transparent transmission of G.711 faxes.

The end-to-end delay for fax signals cannot exceed two seconds.

IAD converts voice calls in G.729 encoding modes into G.711 faxes for transparent transmission.

## Dual-Homing

IAD supports dual-homing for Session Border Controllers (SBCs), SIP servers, and MGC servers.

## Cascading Function

IAD can be connect to another IAD through the LAN port. The cascading function can be used to expand the capacity. IAD132E(T) and IAD1224/IAD196 support three-level cascading.

## Hotline Services

Hotline services allow a user to be connected automatically to a preset phone number (hotline number) after the user picks up the phone. IAD supports instant hotline and delayed hotline services.

Instant hotline: A user is immediately connected to a preset phone number (hotline number) after the user picks up the phone.

Delayed hotline: A user is connected to a preset phone number (hotline number) if the user does not perform any operations within the specified duration (such as five seconds) after the user picks up the phone.

IAD supports hotline services when deployed with NGN or U1900.

## Local Switch

IAD allows POTS users to make calls to each other by dialing numbers allocated by IAD. When the local switch function is enabled, numbers allocated by the NGN, IMS, or softswitch cannot be used. This function can be enabled or disabled on the command line interface (CLI).

## Local Switch When Network Connection Is Normal

IAD allows users to make calls to each other by dialing short numbers when the network connection is normal. The signaling is not transmitted through the service. Therefore, only the basic call function is available.

## Polarity Reversal Charging

IAD sends polarity reversal signals to a charging system. The charging system starts charging when the polarity reversal signals are detected.

Polarity reversal refers to the phenomenon in which a voltage instantaneously changes from positive to negative or from negative to positive.

## Video Call Fallback

Users under IAD can make voice calls with video terminal users. If the signaling contains video information, IAD can recognize the information.

## MWI

When a user has a message, the server sends a notification to IAD. IAD turns on the message waiting indicator (MWI) of the corresponding phone. IAD supports the MWI function when deployed with the NGN.

## Power-off Survival

When IAD encounters a power failure, the relay inside IAD is closed to allow FXS ports that support the power survival function to be connected to FXO ports physically. Phones connected to these FXS ports can still make or answer PSTN calls. One FXS port maps only one FXO port.

IAD132E(T)-8O24S contains two boards. One board provides eight FXO ports and eight FXS ports. The other board provides 16 FXS ports. Only FXS ports on the first board support the power-off survival function. On IAD1224/IAD196, only FXS ports on the board with 12 FXO ports and 12 FXS ports support the power-off survival function.

## Network Interruption Survival

When IAD that uses a static IP address disconnects from the softswitch, IAD automatically enters the local switch mode. In local switch mode, users under IAD can make calls to other intra-office IAD users. If IAD has FXO ports, users can make outer-office calls through the IAD.

IAD1224/IAD196 supports the local switch function in the DHCP or PPPoE mode.

In the local switch mode, IAD supports only basic voice calls.

## Session Authentication

Session authentication refers to authentication that occurs during a session when a terminal changes from static to activated state or terminals are switched. IAD supports the session authentication function when deployed with the IMS.

## Hookflash Function

If a user presses the hookflash and releases it within 200 ms during a call, IAD does not consider it as a hang up operation. Instead, the operation is considered as a call hold operation. The call is ended only when the user presses and holds the hookflash for more than 200 ms.

The typical application of the hookflash is call transfer. When a user wants to transfer a call to another extension number, the user can press the hookflash and dial the extension number. IAD holds the call and connects the call to the extension number.

## Three-Party Services

The three-party services include the three-party conversation and three-party conference. If IAD reports an invite message to the softswitch (for example, when the IAD is connected to the IMS), you must configure IAD to implement three-party services; if the IAD reports an info message to the softswitch (for example, when the IAD is connected to U19100), you only need to configure the softswitch to implement three-party services.

Three-party services can be provided by IAD132/196/1224 or by the IMS. When Three way mode is set to local, you can use the three-party services on IAD no matter whether the services are enabled on the IMS.

## Emergency Call

Users can configure emergency numbers on IAD. When IAD is registered with a server and a user dials an emergency number, IAD directly sends the call request to the server. A user can dial an emergency number even when IAD is not registered with a server. In this case, IAD can still send the call request. IAD supports the emergency call function when deployed with the NGN.

## Digitmap Function

A digitmap specifies a dialing rule. When a user dials a number, if the dialed number length is the same as that defined in the digitmap, IAD stops collecting digits and initiates the call, which reduces the connection duration. If the dialed number does not match the digitmap, the call cannot be connected.

The maximum length of a digitmap is 4096 bytes. When the SIP protocol is used, a maximum of 128 digitmaps can be configured.

## Multi-Country Adaptation

You can change the voice configuration of the device by setting the area information. For example, if you set the country to United States, the system will play announcements in English. At this time, the American busy tone standard is used for FXO ports.

The following countries are supported: Brazil, Chile, China Mainland, Colombia, Egypt, Great Britain, Hong Kong, Russia Kingdom, Singapore, and United States.

The device provides default FXO port busy tone settings for China (China Mainland and Hong Kong), Russia, and Brazil. For the devices delivered to countries excluding China, Russia, and Brazil, the default China settings are configured before delivery. You can define the following parameters: busy tone frequency (single-frequency or dual-frequency), busy tone duration, minimum duration that a busy tone stays idle, and lower limit of the low frequency's level.

## Phone Port Properties

IAD allows you to adjust the gain for phone ports, and supports pulse dialing, DTMF dialing, and polarity reversal pulses.

## VLAN Function

IAD supports the VLAN configuration function, including assigning VLANs for ports and adding different VLAN tags to different types of packets to ensure the voice quality or data quality preferentially when the network condition is not good.

## VQM Function

IAD132/196/1224 supports the VQM function, which monitors the voice quality in real time and sends the quality information to a data center.

## Signaling and Media Encryption

IAD1224/IAD196 supports Transport Layer Security (TLS) encryption V1.1 and media stream encryption.

## Service List (with IMS)

| Service | Sub-service |
| --- | --- |
| Registration functions | SBC dual-homing |
|  | SBC switching |
|  | SBC switching back |
|  | User-by-user registration/Refreshing registration/Deregistration |
|  | Wildcard registration/Refreshing registration/Deregistration |
| Basic voice services | Voice intercommunication between IAD users and other IMS users |
| Supplementary services | Call-out restriction service |
|  | DND service |
|  | Alarm call |
|  | CFU service |
|  | CFB service |
|  | CFNR service |
|  | Call forwarding unregistered |
|  | Call forwarding times (only twice) |
|  | Blind call transfer |
|  | Query transfer |
|  | CLIP service |
|  | CLIR service |
|  | CLIR override |
|  | Three-party conversion (terminal audio mixing) |
|  | CRBT service |
| Unified Centrex | Short number dialing |
|  | Same-group call pickup |
|  | Designated call pickup service |
|  | Displaying extension long numbers for outgoing calls |
|  | Displaying the main number for outgoing calls |
|  | Displaying the main number and short numbers of intra-group extensions for outgoing calls |
|  | Call hold of inter-IMS users |
|  | Call wait of inter-IMS users |
|  | Distinctive ringing |
| Fax services | G711 transparent transmission mode |
|  | Fax in VBD transparently-transmitted mode |
|  | T.38 fax |
|  | Switch from G.729 to T.38 |
|  | Switch from G.729 to G.711 fax transparent transmission |
| Charging service | T01-0901 polarity reversal charging |
| Security function | Session authentication |
| Performance statistics services | Statistics on successful incoming calls |
|  | Statistics on successful outgoing calls |
|  | CPU usage |

## Service List (with NGN)

| Service | Sub-service | MGCP | SIP |
| --- | --- | --- | --- |
| Registration function | Registration/Refreshing registration/Deregistration | √ | √ |
| Basic voice services | Local call service | √ | √ |
|  | Toll call service | √ | √ |
|  | Long number intercommunication between IADs | √ | √ |
|  | Intercommunication between IADs on the virtual network | √ | √ |
| Supplementary services | Abbreviated dialing service | √ | √ |
|  | Hotline service | √ | √ |
|  | Call-out restriction service | √ | √ |
|  | DND service | √ | √ |
|  | Hookflash service | √ | √ |
|  | Alarm clock service | √ | √ |
|  | CFU service | √ | √ |
|  | CFB service | √ | √ |
|  | CFNR service | √ | √ |
|  | Absent user service | √ | √ |
|  | Call waiting service | √ | √ |
|  | Call back on busy service | √ | x |
|  | Three-party conversation service | √ | √ |
|  | Voice conference service | √ | √ |
|  | Designated pickup service | √ | √ |
|  | Call transfer service | √ | √ |
|  | CLIP service | √ | √ |
|  | CLIR service | √ | √ |
|  | Secretary service | √ | x |
|  | Secretary station service | √ | x |
|  | RCB service | √ | x |
|  | Call holding service | √ | √ |
|  | Emergency call service | √ | √ |
|  | MWI service | √ | √ |
| Fax services | G711 transparent transmission service | √ | √ |
|  | T.38 fax service | √ | √ |
|  | Local fax service | √ | √ |
|  | Toll fax service | √ | √ |
| Charging services | Polarity reversal charging | √ | √ |
|  | 12KC charging | √ | x |
|  | 16KC charging | √ | x |

## Service List (with U1900)

| Service | Sub-service |
| --- | --- |
| Registration function | User registration |
| Basic voice services | Basic voice service between IAD users and other SoftCo users |
| Supplementary services | Local number querying service |
|  | CLIP service |
|  | CLIR service |
|  | CFU service |
|  | CFNR service |
|  | CFB service |
|  | CFO service |
|  | Blind call transfer |
|  | Semi-query transfer |
|  | Query transfer |
|  | Hold service |
|  | Call Park service |
|  | CW service |
|  | Three-party service |
|  | CONF service |
|  | Abbreviated dialing service |
|  | Co-group pickup service |
|  | Designated pickup service |
|  | Absent user service |
|  | DND service |
|  | Hotline service |
|  | Alarm clock service |
|  | Call-out restriction service |
|  | RCB service |
|  | Call back on busy service |
|  | Secretary services |
|  | Privilege user services |
|  | Simultaneous ringing service |
|  | Sequential ringing service |
|  | Distinctive ring tone service |
|  | ONLY service |
|  | NP service |
| Fax services | G.711 faxes transmitted transparently |
|  | T.38 faxes transmitted transparently |